# Arduino Uno R4 Lesson: Using the HC-SR04 Ultrasonic Sensor

## Lesson Overview

This lesson teaches students how to use the HC-SR04 ultrasonic sensor with the Arduino Uno R4 to measure distance using sound waves. Students will learn wiring, timing-based measurement, and how to display distance readings in the Serial Monitor.

## Learning Objectives

• Explain how the HC-SR04 ultrasonic sensor measures distance.

• Wire the trigger and echo pins to the Arduino Uno R4.

• Use pulse timing to calculate distance in centimeters.

• Display distance readings using the Serial Monitor.

• Modify the code for applications like obstacle detection.

## Materials Needed

• Arduino Uno R4

• HC-SR04 Ultrasonic Sensor

• USB-C cable

• Jumper wires

• Breadboard (optional)

• Arduino IDE installed

## Section 1: How the HC-SR04 Works

The HC-SR04 uses ultrasonic sound waves to measure distance. It sends a brief trigger pulse, waits for the sound to bounce off an object, and measures the time it takes to return. The Arduino converts this time into a distance measurement.

## Section 2: Wiring the Sensor

Connect the HC-SR04 ultrasonic sensor to the Arduino Uno R4 as follows:

• VCC → 5V

• GND → GND

• TRIG → Pin 9

• ECHO → Pin 10

## Section 3: Distance Measurement Code

Upload the following example code to measure distance and display it in the Serial Monitor:

// HC-SR04 Ultrasonic Sensor Example
// Arduino Uno R4

const int trigPin = 9;
const int echoPin = 10;

void setup() {
  Serial.begin(9600);
  pinMode(trigPin, OUTPUT);
  pinMode(echoPin, INPUT);
}

void loop() {
  digitalWrite(trigPin, LOW);
  delayMicroseconds(2);

  digitalWrite(trigPin, HIGH);
  delayMicroseconds(10);
  digitalWrite(trigPin, LOW);

  long duration = pulseIn(echoPin, HIGH);
  long distance = duration / 58;

  Serial.print("Distance: ");
  Serial.print(distance);
  Serial.println(" cm");

  delay(200);
}

## Section 4: Student Activities

• Activity 1: Modify the delay to read distance once per second.

• Activity 2: Print a warning message if distance is less than 10 cm.

• Activity 3: Add an LED on pin 3 that lights up when distance is less than 15 cm.

• Activity 4: Create a parking sensor that prints “Clear”, “Approaching”, or “STOP” based on distance.

## Section 5: Real-World Applications

• Robotics obstacle avoidance

• Parking assist systems

• Automatic doors

• Smart trash bins

• Security and motion detection systems