# Intro to Arduino Uno R4: Beginner Lesson Plan

## Lesson Overview

This lesson introduces students with no prior programming experience to Arduino programming using the Arduino Uno R4. Students will learn the basics of C/C++-style sketches, digital output, digital input, and how to upload programs.

## Learning Objectives

• Understand what the Arduino Uno R4 is.

• Describe the structure of an Arduino sketch.

• Upload a sketch using the Arduino IDE.

• Control an LED using digital output.

• Use a button as digital input.

• Modify example programs to change behavior.

## Materials Needed

• Arduino Uno R4 (WiFi or Minima)

• USB‑C cable

• Computer with Arduino IDE installed

• LED and 220–330 Ω resistor

• Pushbutton

• Breadboard and jumper wires

## Section 1: What is Arduino?

Arduino is a small programmable microcontroller platform used to create interactive electronics. The Uno R4 can read sensors, control actuators, blink LEDs, and run code continuously.

## Section 2: What Language Does Arduino Use?

Arduino uses a simplified version of the C/C++ language. Every Arduino sketch contains two required functions:

void setup() {
    // Runs once when powered on
}

void loop() {
    // Runs repeatedly
}

The setup() function initializes the board; loop() runs continuously for tasks like blinking LEDs or reading sensors.

## Section 3: First Program – Blinking an LED

The Arduino Uno R4 includes a built‑in LED on pin 13. This is the perfect first project.

Enter the following code into the Arduino IDE:

void setup() {
  pinMode(13, OUTPUT);  // Set pin 13 as an output
}

void loop() {
  digitalWrite(13, HIGH);  // Turn LED on
  delay(1000);             // Wait 1 second
  digitalWrite(13, LOW);   // Turn LED off
  delay(1000);             // Wait 1 second
}

Upload the code. The LED should blink once per second.

## Activity 1: Modify the Blink Pattern

• Change the LED to blink twice as fast.

• Create a long–short–long Morse‑style pattern.

• Blink three times quickly, pause, then repeat.

## Section 4: Reading a Button Input

Students will wire a button to pin 2 and modify the sketch to turn the LED on when the button is pressed.

Example code:

int buttonPin = 2;

void setup() {
  pinMode(13, OUTPUT);
  pinMode(buttonPin, INPUT);
}

void loop() {
  int state = digitalRead(buttonPin);

  if (state == HIGH) {
    digitalWrite(13, HIGH);  // LED on
  } else {
    digitalWrite(13, LOW);   // LED off
  }
}

## Activity 2: Button Challenges

• Make the LED turn on when the button is NOT pressed.

• Make the LED blink while the button is held.

• Blink faster the longer the button is held.

• Create a version where one press keeps the LED on for 3 seconds.

## Check for Understanding

• What does pinMode() do?

• Explain the difference between setup() and loop().

• What does digitalWrite() do?

• What happens if delay() is removed from the blink program?

## Next Steps for Students

• Using sensors (temperature, ultrasonic, light).

• Controlling servos or motors.

• Using analog input with analogRead().

• Building simple robots.

• Writing custom reusable functions.